Upgrade firmware SOP:

Connect to switch's console port and management port from a PC

Create a folder on the desktop of PC and unzip the firmware file into the folder. There are three files

FW: XGS-6350-24X4C_2.2.0D_158138.bin

Web UI: web_PLANET_XGS-6350-24X4C.wrp

Execute Tftpd64 utility and configure "Current Directory" and "Server interfaces"

Execute Tera Term or putty to enter command line

Note: Set the PC's ip address to the 192.168.1.x (ex. 192.168.1.29) network segment

（1）Upgrade FW

Once ”SDRAM Fast Test……..PASS “ is displayed after boot, press Ctrl+p to enter monitor mode.

monitor#ip add 192.168.1.22 255.255.255.0 ----------------Set IP Address for the switch

monitor#copy tftp flash

Source file name[]?XGS-6350-24X4C_2.2.0D_158138.bin

Remote-server ip address[]?192.168.1.29

Destination file name[XGS-6350-48X2Q4C_2.2.0D_110610.bin]?switch.bin

（2）Upgrade Web UI

monitor#copy tftp flash

Source file name[]?web_PLANET_XGS-6350-24X4C.wrp

Remote-server ip address[]?192.168.1.29

Destination file name[web.wrp]?web.wrp

monitor#reboot

Do you want to reboot the Switch(y/n)?y