XL-GHW201x System Firmware Release Note

Version 3.94     2024/12/03

Following is a list of new features and bugs fixed introduced in version 3.94 of the EoC Coax MDU system firmware relative to version 3.93

1. Misc.

|  | Description |
| --- | --- |
| 1 | Syslog and Web will prompt EP “loop” warning while XL-GH201(since EoC driver v3.2.4) (since WiFi driver v1.4.01) has detected loop and closed its Ethernet port automatically. |

Version 3.93     2024/11/05

Following is a list of new features and bugs fixed introduced in version 3.93 of the EoC Coax MDU system firmware relative to version 3.92

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug to send PAUSE toward CEM8392 EoC2 to throttle the upstream traffic, the NMU will not transmit PAUSE toward EoC1 & EoC2 afterward. |

Version 3.92     2024/10/15

Following is a list of new features and bugs fixed introduced in version 3.92 of the EoC Coax MDU system firmware relative to version 3.90

1. Misc.

|  | Description |
| --- | --- |
| 1 | To support IPv4 IGMP and IPv6 MLD snooping function, there are 3 operating modes: 1) Disable (by default) IGMP+MLD snooping 2) Enable IGMP+MLD snooping, and block unknown multicast traffic 3) Enable IGMP+MLD snooping, and forward unknown multicast traffic |

2. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug that EP has wrong link status indication (as Master Web displays a yellow “Link” LED icon for EP). To restart and re-configure EP will get this trouble after Master (NMU) goes online for more than 50 days. |

Version 3.90     2024/09/25

Following is a list of new features and bugs fixed introduced in version 3.90 of the EoC Coax MDU system firmware relative to version 3.89

1. Misc.

|  | Description |
| --- | --- |
| 1 | For WiFi EP VAP WPA-Enterprise (RADIUS) mode, to add extra settings of RADIUS “Auth” port number & “Accounting” port number. |

Version 3.89     2024/08/12

Following is a list of new features and bugs fixed introduced in version 3.89 of the EoC Coax MDU system firmware relative to version 3.88

1. Misc.

|  | Description |
| --- | --- |
| 1 | To support new band scan function, normally the low band starts at 0 MHz and the whole working band is up to 200MHz. Some operators request to reserve bands below 70MHz for Wave1 devices or others, now the low band could start at 75MHz, to make 75~200MHz the working band (Master and EP loaded with EoC driver v3.2.2j is mandatory, marked with extra “j” version). If firmware detects its Master EoC driver is “j” version, as now the v3.2.2j, it will allow a “band shift” setting, operator may choose either 0MHz (default) or 75MHz to shift. The EP devices loaded with v3.2.2j will search the right low band starting point (0 or 75MHz) and link at the right starting low band automatically. Note if EP is loaded EoC with none “j” version, as v3.2.1, EP will lose link after Master(v3.2.2j) shifts the low band to 75MHz, link will regain only if Master(v3.2.2j) shifts back to 0MHz. |

Version 3.88     2024/07/29

Following is a list of new features and bugs fixed introduced in version 3.88 of the EoC Coax MDU system firmware relative to version 3.85

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug that management packets between Master and EP may be lost occasionally. |

Version 3.85     2024/07/05

Following is a list of new features and bugs fixed introduced in version 3.85 of the EoC Coax MDU system firmware relative to version 3.84

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug in (WiFi EP) Lan1/Lan2 port “Host Limit”. Also to close the not working WiFi port “Host Limit” function. There are two methods to block wireless client from joining WiFi VAP, 1) Close “Radio 2.4G&5G WiFi Service” 2) Disable all “VAP_2.4G & VAP_5G VAP1/2/3/4 Service” |

Version 3.84     2024/06/11

Following is a list of new features and bugs fixed introduced in version 3.84 of the EoC Coax MDU system firmware relative to version 3.83

1. Misc.

|  | Description |
| --- | --- |
| 1 | (WiFi EP) VAP supports WPA-Enterprise (RADIUS) mode, besides original WPA-Personal mode. The extra settings of RADIUS “Server IP” & “Shared Secret” is mandatory for Enterprise mode. |

Version 3.83     2024/05/27

Following is a list of new features and bugs fixed introduced in version 3.83 of the EoC Coax MDU system firmware relative to version 3.82

1. Misc.

|  | Description |
| --- | --- |
| 1 | To change the global “Hide SSID” VAP setting of WiFi EP into a local VAP setting, each VAP could have its own “Hide SSID” option to hide/display self SSID. |

Version 3.82     2024/04/23

Following is a list of new features and bugs fixed introduced in version 3.82 of the EoC Coax MDU system firmware relative to version 3.81

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug with incorrect print format in Syslog. |

Version 3.81     2024/02/25

Following is a list of new features and bugs fixed introduced in version 3.81 of the EoC Coax MDU system firmware relative to version 3.80

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug to change EP WiFi AP “Security Mode” from “open” mode to other mode(as WPA3) will fail. Both “User Defined EP Default Profile” and EP current configuration profile have this bug. |

Version 3.80     2024/02/27

Following is a list of new features and bugs fixed introduced in version 3.80 of the EoC Coax MDU system firmware relative to version 3.79

1. Misc.

|  | Description |
| --- | --- |
| 1 | A refinement of v3.79. Even downgrade the system firmware from v3.80(or above) to v3.78(or below) will also fix the previous v3.79 bug. |

Version 3.79     2024/02/22

Following is a list of new features and bugs fixed introduced in version 3.79 of the EoC Coax MDU system firmware relative to version 3.78

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug to restore the EP backup file that may recover the wrong “Host Limit” setting to unsupported Coax port. That will disconnect clients at EP from Internet connection. |

Version 3.78     2024/02/05

Following is a list of new features and bugs fixed introduced in version 3.78 of the EoC Coax MDU system firmware relative to version 3.77

1. Misc.

|  | Description |
| --- | --- |
| 1 | While "Allow user to configure WiFi" is changed to "Disable" within "User Defined EP Default Profile", its VAP "SSID"/"Pre-Shared key" could apply to WiFi EP as default settings. In order to have a common/global VAP "SSID"/"Pre-Shared key" for all WiFi EP in advance. Note this "Disable" will also overwrite WiFi EP built-in factory default "SSID"/"Pre-Shared key" (printing on back label). |

Version 3.77     2024/01/17

Following is a list of new features and bugs fixed introduced in version 3.77 of the EoC Coax MDU system firmware relative to version 3.76

1. Misc.

|  | Description |
| --- | --- |
| 1 | To add logout function(icon) on Web. |
| 2 | To display customized logo on Web according to factory default Customer ID. |

Version 3.76     2023/12/08

Following is a list of new features and bugs fixed introduced in version 3.76 of the EoC Coax MDU system firmware relative to version 3.75

1. Misc.

|  | Description |
| --- | --- |
| 1 | To add scheduling time to upgrade Master/EP EoC driver within “Auto Provision”. To prearrange the interrupted service period to update driver automatically. |
| 2 | For (WiFi EP) default NAT mode, its WAN interface will not response to Ping (by iptables). |

2. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug to restore the backup CEM839 Master EoC settings (Note/Notch…) correctly. |

Version 3.75     2023/11/28

Following is a list of new features and bugs fixed introduced in version 3.75 of the EoC Coax MDU system firmware relative to version 3.74

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix an IPv6 multicast bug, the multicast packet (33-33-xx-xx-xx-xx) should not be throttled while IGMP is not enabled. |

Version 3.74     2023/11/01

Following is a list of new features and bugs fixed introduced in version 3.74 of the EoC Coax MDU system firmware relative to version 3.73

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug to accept character “\” in “Directory Path” string of “Auto Provision” setting. |

Version 3.73     2023/10/03

Following is a list of new features and bugs fixed introduced in version 3.73 of the EoC Coax MDU system firmware relative to version 3.71

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug that may cause watchdog reboot if “Auto Provision” is running. |

Version 3.71     2023/09/25

Following is a list of new features and bugs fixed introduced in version 3.71 of the EoC Coax MDU system firmware relative to version 3.70

1. Misc.

|  | Description |
| --- | --- |
| 1 | To eliminate the EP EoC port "Host Limit" setting, alike CEM838/837. This setting conflicts with multicast stream & IGMP setup, they could not co-exist. The EP "Host Limit" function works on EP Lan1/Lan2/WiFi port, it doesn't work on EP EoC port. |

2. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug while EP VLAN setting is changed. Issue EP switch software reset before re-configuring(RECFG) EP to clear cached EP setting residues. |

Version 3.70     2023/09/06

Following is a list of new features and bugs fixed introduced in version 3.70 of the EoC Coax MDU system firmware relative to version 3.64

1. Misc.

|  | Description |
| --- | --- |
| 1 | Rewind Master parameter NODE.GENERAL.DOMAIN_ID_MODE=1 for compatibility, to connect EP with EoC driver v2.x.x and v3.x.x. |

Version 3.64     2023/08/30

Following is a list of new features and bugs fixed introduced in version 3.64 of the EoC Coax MDU system firmware relative to version 3.63

1. Misc.

|  | Description |
| --- | --- |
| 1 | Support SNR/PSD connection status diagnosis via layer2 protocol, not have to assign Master EoC device & EP EoC device layer3 IP address in advance in order to measure SNR/PSD (just keep IP address as default 0.0.0.0). |

2. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug while upgrading boot loader. |

Version 3.63     2023/08/03

Following is a list of new features and bugs fixed introduced in version 3.63 of the EoC Coax MDU system firmware relative to version 3.62

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a model mismatched bug while upgrading EoC driver. |

Version 3.62     2023/07/10

Following is a list of new features and bugs fixed introduced in version 3.62 of the EoC Coax MDU system firmware relative to version 3.61

1. Misc.

|  | Description |
| --- | --- |
| 1 | Co-work with EoC Master & EP driver v2.0.8j. To add vendor notches 70~200MHz for XL-GHW201J & XL-GH201J & J. |

Version 3.61     2023/06/30

Following is a list of new features and bugs fixed introduced in version 3.61 of the EoC Coax MDU system firmware relative to version 3.60

1. Misc.

|  | Description |
| --- | --- |
| 1 | Co-work with EoC Master & EP driver v2.0.6j. To add vendor notches 70~200MHz for XL-GHW201J & XL-GH201J. |
| 2 | To refine the PSD display diagram while doing diagnosis via Web browser. |

Version 3.60     2023/02/09

Following is a list of new features and bugs fixed introduced in version 3.60 of the EoC Coax MDU system firmware relative to version 3.50

1. Misc.

|  | Description |
| --- | --- |
| 1 | Co-work with EoC Master & EP driver v3.1.0. To set up EP parameter NODE.GENERAL.DOMAIN_ID_MODE=6 for EP with EoC v2.x.x. |

Version 3.50     2022/11/22

Following is a list of new features and bugs fixed introduced in version 3.50 of the EoC Coax MDU system firmware relative to version 3.49

1. Misc.

|  | Description |
| --- | --- |
| 1 | Make the “LED Dark Mode” a global setting within CEM839/8392, all EPs share this same setting. All EPs’ LED are either dark or lighting simultaneously. Master’s LED remains lighting normally. |
| 2 | To change the CEM8392 (dual Master) default setting “Active Second Master” from “Off” to “On”. That is, it will enable two Masters to support up to maximum 32 EPs by default. The previous default is “Off”, that activates only one Master and supports up to 16 EPs. Note this will affect the packet latency, since two active Masters share the bandwidth evenly and each Master will get just half. May change this to “Off” if you intend to explore/test the latency/throughput by connecting just one or few EP. |

2. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | To reconfigure “RECFG” EP may take longer than expected, while web GUI may response annoying “Can not access device…” message. |

Version 3.49     2022/11/18

Following is a list of new features and bugs fixed introduced in version 3.49 of the EoC Coax MDU system firmware relative to version 3.47

1. Misc.

|  | Description |
| --- | --- |
| 1 | To support EP(XL-GH201) & WiFi EP () “LED Dark Mode”, set this mode to “On” by Master(CEM839/8392) will turn off all EP’s LED. EP is running without any LED indication/blinking. In order to run the device in no-lights environment. The default mode is “Off” that EP lights/blinks LED normally. |
| 2 | To refine the constraint that there will exist only one superuser and its username “admin” is unchangeable. |

Version 3.47     2022/06/09

Following is a list of new features and bugs fixed introduced in version 3.47 of the EoC Coax MDU system firmware relative to version 3.46

1. Misc.

|  | Description |
| --- | --- |
| 1 | To refine the EP EoC driver update procedure to display the new driver version info immediately after EP warm reboot. |

Version 3.46     2022/04/19

Following is a list of new features and bugs fixed introduced in version 3.46 of the EoC Coax MDU system firmware relative to version 3.45

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | WiFi EP() LAN1/LAN2 port will output egress tagged packet in default “NAT” network mode. This bug is fixed by changing “untouched” mode into “untagged” mode of EP LAN1/LAN2 port. |

Version 3.45     2022/04/07

Following is a list of new features and bugs fixed introduced in version 3.45 of the EoC Coax MDU system firmware relative to version 3.44

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | To fix a bug in “Auto Upgrade EoC Driver” function to upgrade/downgrade online Master & EP EoC driver automatically. |

Version 3.44     2022/02/14

Following is a list of new features and bugs fixed introduced in version 3.44 of the EoC Coax MDU system firmware relative to version 3.43

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | To fix a bug to lose WiFi EP() VAP configuration. |

Version 3.43     2022/01/12

Following is a list of new features and bugs fixed introduced in version 3.43 of the EoC Coax MDU system firmware relative to version 3.42

1. Misc.

|  | Description |
| --- | --- |
| 1 | The WiFi EP() supports new “VAP with VLAN” bridging mode. Each Virtual AP could be assigned with one VLAN ID and be treated as one tagged WiFi interface. This new operation mode that differs from the original simple VLAN mode that will assign all WiFi VAPs’ clients with the same WiFi VLAN ID. For example, VAP1 could be assigned with VID=201 and VAP2 could be 202, then the tagged VID=201 packet goes to(from) VAP1 and the tagged VID=202 packet goes to(from) VAP2. |

Version 3.42     2022/01/04

Following is a list of new features and bugs fixed introduced in version 3.42 of the EoC Coax MDU system firmware relative to version 3.41

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | To fix a bug to miscalculate telnet sessions. |

Version 3.41     2021/12/02

Following is a list of new features and bugs fixed introduced in version 3.41 of the EoC Coax MDU system firmware relative to version 3.40

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | To fix a bug that WiFi EP() WiFi “port rate limit” failed to work while it is enabled. |
| 2 | To be in sync with WiFi EP() driver V1.3.06. To fix a bug to keep the egress tagged packet untainted while Tag VLAN is not enabled(change unmodified mode into untouched mode) by default. |

Version 3.40     2021/11/12

Following is a list of new features and bugs fixed introduced in version 3.40 of the EoC Coax MDU system firmware relative to version 3.39

1. Misc.

|  | Description |
| --- | --- |
| 1 | To add telnet command “swcascade” to setup Master into cascaded mode. Note that the management device at the down-link side could not access the cascaded Master (NMU), the cascaded Master will response to the up-link side only. |

Version 3.39     2021/09/10

Following is a list of new features and bugs fixed introduced in version 3.39 of the EoC Coax MDU system firmware relative to version 3.38

1. Misc.

|  | Description |
| --- | --- |
| 1 | The WiFi EP() security mode supports “WPA2 only”. |
| 2 | To revise telnet commands “mascoax” & “epcoax”, to add IP address subnet mask option by “-s” |
| 3 | To revise telnet command “masnotch”, to add option “-f” to delete all “User Defined Notches”. |

Version 3.38     2021/07/23

Following is a list of new features and bugs fixed introduced in version 3.38 of the EoC Coax MDU system firmware relative to version 3.37

1. Misc.

|  | Description |
| --- | --- |
| 1 | To support new NTC temperature sensor. |

Version 3.37     2021/05/26

Following is a list of new features and bugs fixed introduced in version 3.37 of the EoC Coax MDU system firmware relative to version 3.36

1. Misc.

|  | Description |
| --- | --- |
| 1 | To adjust the ARP packet priority of idle EP. |

Version 3.36     2021/05/19

Following is a list of new features and bugs fixed introduced in version 3.36 of the EoC Coax MDU system firmware relative to version 3.35

1. Misc.

|  | Description |
| --- | --- |
| 1 | To revise telnet command “rmep -m all” to delete all off-line EP profiles at once. |

Version 3.35     2021/05/06

Following is a list of new features and bugs fixed introduced in version 3.35 of the EoC Coax MDU system firmware relative to version 3.32

1. Misc.

|  | Description |
| --- | --- |
| 1 | To ease the EP statistic per-minute-polling load. |
| 2 | To pad one more parameter into user notch setting, to degrade 1~40dB(adjustable) signal power, by default max=100dB to remove carriers(turn off the band) |

2. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | To fix a bug that auto-configure the user notch. |
| 2 | To fix telnet command “swconfig” error while processing eoc port argument. |
| 3 | The auto-configuration commands “masdefault” & “masnotch” should belong to Master *.dev profile, instead of *.nmu profile. |

Version 3.34j(Optional)     2021/04/26

Following is a list of new features and bugs fixed introduced in version 3.34j of the EoC Coax MDU system firmware relative to version 3.32

1. Misc.

|  | Description |
| --- | --- |
| 1 | To block the egress broadcast packet on EP LAN2 port. This will prevent the broadcast packet (as ARP) from flooding toward EP LAN2 port. (RTM.GENERAL.UNKNOWN_SUPPRESSION=NO as version 3.32) |

Version 3.33j(Optional)     2021/04/19

Following is a list of new features and bugs fixed introduced in version 3.33j of the EoC Coax MDU system firmware relative to version 3.32

1. Misc.

|  | Description |
| --- | --- |
| 1 | To block the unlearned destination MAC packet forwarded to EoC coax. This will set RTM.GENERAL.UNKNOWN_SUPPRESSION=YES. |

Version 3.32     2021/04/16

Following is a list of new features and bugs fixed introduced in version 3.32 of the EoC Coax MDU system firmware relative to version 3.31

1. Misc.

|  | Description |
| --- | --- |
| 1 | To extend the WiFi EP() WPA key(password) length from 8~15 characters to 8~63 characters. It cooperates with WiFi driver version 1.3.04. |

Version 3.31     2021/02/25

Following is a list of new features and bugs fixed introduced in version 3.31 of the EoC Coax MDU system firmware relative to version 3.30

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug while configuring WiFi EP() VAP 2.4G/5G. |

Version 3.30     2021/02/09

Following is a list of new features and bugs fixed introduced in version 3.30 of the EoC Coax MDU system firmware relative to version 3.20

1. Misc.

|  | Description |
| --- | --- |
| 1 | WiFi EP() supports WPA3. |
| 2 | Switch runs IVL(Independent VLAN Learning) mode, instead of SVL(Shared VLAN Learning) mode. |

Version 3.20     2020/11/20

Following is a list of new features and bugs fixed introduced in version 3.20 of the EoC Coax MDU system firmware relative to version 3.19

1. Misc.

|  | Description |
| --- | --- |
| 1 | To show receiver PSD (Power Spectral Density) diagram on diagnosis web page, besides the original SNR diagram. |
| 2 | WiFi EP() could be configured to meet Australia WiFi frequency bands in advance. |

Version 3.19     2020/07/09

Following is a list of new features and bugs fixed introduced in version 3.19 of the EoC Coax MDU system firmware relative to version 3.17

1. Misc.

|  | Description |
| --- | --- |
| 1 | To revise SNMP OID and MIB file. |

Version 3.17     2020/05/15

Following is a list of new features and bugs fixed introduced in version 3.17 of the EoC Coax MDU system firmware relative to version 3.16

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a WiFi EP() VLAN membership bug while both VLAN and NAT are enabled. |

Version 3.16     2020/05/05

Following is a list of new features and bugs fixed introduced in version 3.16 of the EoC Coax MDU system firmware relative to version 3.15

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a DHCP snooping bug while VLAN is enabled(MAC address should apply SVL). |

Version 3.15     2020/03/06

Following is a list of new features and bugs fixed introduced in version 3.15 of the EoC Coax MDU system firmware relative to version 3.14

1. Misc.

|  | Description |
| --- | --- |
| 1 | To sync all Master/EP IGMP snooping settings on EoC. |

2. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a bug while programming user notch filter. |

Version 3.14     2020/02/06

Following is a list of new features and bugs fixed introduced in version 3.14 of the EoC Coax MDU system firmware relative to version 3.13

1. Misc.

|  | Description |
| --- | --- |
| 1 | To bind on-line EP EoC MAC address to Master device to handle EP loop back issue. |

Version 3.13     2020/01/13

Following is a list of new features and bugs fixed introduced in version 3.13 of the EoC Coax MDU system firmware relative to version 3.10

1. Misc.

|  | Description |
| --- | --- |
| 1 | Co-work with EoC Master & EP driver v2.0.3 (CUSID=113, with extra vendor notches, the running frequency band spans over 110MHz~200MHz only) |

2. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Reboot CEM8392 in order to activate or deactivate the 2nd Master should wait longer till the settings rewriting is complete. |

Version 3.10     2019/12/26

Following is a list of new features and bugs fixed introduced in version 3.10 of the EoC Coax MDU system firmware relative to version 3.05

1. Misc.

|  | Description |
| --- | --- |
| 1 | Co-work with EoC Master & EP driver v2.0.0. |

2. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a Master full flash upgrade bug. |

Version 3.05     2019/12/10

Following is a list of new features and bugs fixed introduced in version 3.05 of the EoC Coax MDU system firmware relative to version 3.04

1. Misc.

|  | Description |
| --- | --- |
| 1 | To disable IGMP snooping for Master and EP by default. |

Version 3.04     2019/10/24

Following is a list of new features and bugs fixed introduced in version 3.04 of the EoC Coax MDU system firmware relative to version 3.03

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | For IGMPv2 snooping, use RTM.GENERAL.UNKNOWN_SUPPRESSION=NO |

Version 3.03     2019/10/14

Following is a list of new features and bugs fixed introduced in version 3.03 of the EoC Coax MDU system firmware relative to version 3.00

1. New Features

|  | Description |
| --- | --- |
| 1 | For XL-GHW2012, web GUI support “Move Ep from one master to another master” and display all master and Ep on one page. |

2. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | VLAN setting for non-WiFi EP can not work, fix it. |

Version 3.00     2019/07/11

Following is a list of new features and bugs fixed introduced in version 3.00 of the EoC Coax MDU system firmware relative to version 2.5

1. New Features

|  | Description |
| --- | --- |
| 1 | To support new XL-GHW2012 dual Masters, besides XL-GHW201. |
| 2 | For XL-GHW201 and XL-GHW2012, to support new EP, besides the XL-GH201 EP. |

Version 2.06     2019/05/16

Following is a list of new features and bugs fixed introduced in version 2.06 of the EoC Coax MDU system firmware relative to version 2.05

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix inaccurate upstream bandwidth control of Master(NMU) LAN1/LAN2 port. |

Version 2.05     2019/02/20

Following is a list of new features and bugs fixed introduced in version 2.05 of the EoC Coax MDU system firmware relative to version 2.04.

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix an IPv4 DHCP snooping bug. |

Version 2.04     2019/01/08

Following is a list of new features and bugs fixed introduced in version 2.04 of the EoC Coax MDU system firmware relative to version 2.03.

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Support to adjust MAC address aging timer, timer could range between 60 and 7200 seconds, default timer value is 300 seconds. |

Version 2.03     2018/11/28

Following is a list of new features and bugs fixed introduced in version 2.03 of the EoC Coax MDU system firmware relative to version 2.01.

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Support to full-flash upgrade Master & EP EoC driver. |

Version 2.01     2018/09/27

Following is a list of new features and bugs fixed introduced in version 2.01 of the EoC Coax MDU system firmware relative to version 2.00.

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | Fix a rate control bug. |
| 2 | Fix a diagnosis display bug while user notches are applied. |

Version 2.00     2018/08/31

Initial release.

1. Misc.

|  | Description |
| --- | --- |
| 1 | Starts Wave2. |