XL-GHW201x, XL-GH201 EoC Driver Release Note

Version 3.2.4	2024/12/03

Following is a list of new features and bugs fixed introduced in version 3.2.4 of the EoC Coax MDU driver relative to version 3.2.1

1. Misc.

|  | Description |
| --- | --- |
| 1 | To support EP loop detection. EP Lan port will be closed immediately while looping. First to remove the wired loop, then to reboot EP will recover. XL-GH201 needs EoC driver v3.2.4 needs WiFi driver v1.4.01 to support this EP automatic loop detection function. |

Version 3.2.3j	2024/08/15

Following is a list of new features and bugs fixed introduced in version 3.2.3j of the EoC Coax MDU driver relative to version 3.2.2j

1. Bugs Fixed.

|  | Description |
| --- | --- |
| 1 | If user adds User Notch to close some signaling band and the Master low band starts at 75MHz (shift to 75MHz), the unwanted signals near 200MHz may be emitted. To revise default Vendor Notch from 200MHz~300MHz to 199999KHz-~275MHz to close the Notch filter gap to fix the bug. |

Version 3.2.2j	2024/08/12

Following is a list of new features and bugs fixed introduced in version 3.2.2j of the EoC Coax MDU driver relative to version 3.2.1

1. Misc.

|  | Description |
| --- | --- |
| 1 | To support new band scan function, normally the low band starts at 0 MHz and the whole working band is up to 200MHz. Some operators request to reserve bands below 70MHz for Wave1 devices or others, now the low band could start at 75MHz, to make 75~200MHz the working band. EP will search the right starting low band (0 or 75MHz) automatically after reboot or restart. To enable this band scan function, C839x system firmware v3.89(or after) is mandatory. |

Version 3.2.1	2023/09/06

Following is a list of new features and bugs fixed introduced in version 3.2.1 of the EoC Coax MDU driver relative to version 3.2.0

1. Misc.

|  | Description |
| --- | --- |
| 1 | Rewind Master DOMAIN_ID_MODE=1 for compatibility, to connect EP with EoC driver version 2.x.x and 3.x.x. |

Version 3.2.0	2023/08/31

Following is a list of new features and bugs fixed introduced in version 3.2.0 of the EoC Coax MDU driver relative to version 3.1.0

1. Misc.

|  | Description |
| --- | --- |
| 1 | Support SNR/PSD connection status diagnosis via layer2 protocol, not have to assign Master EoC device & EP EoC device layer3 IP address in advance in order to measure SNR/PSD (just keep IP address as default 0.0.0.0). |
| 2 | Please note that XL-GHW201/8392 system firmware version after 3.6x (now is 3.64) is a must to upgrade EoC driver 3.x.x. For previous system firmware (for example 3.5x), please follow the steps: 2.1) Upgrade system firmware first to 3.64 and have all EPs online. 2.2) Upload EoC driver 3.x.x onto XL-GHW201/8392 2.3) Upgrade all EPs’ EoC driver from 2.x.x to 3.x.x 2.4) Upgrade Master EoC driver from 2.x.x to 3.x.x, this is the last step |

Version 3.1.0	2023/02/06

Following is a list of new features and bugs fixed introduced in version 3.1.0 of the EoC Coax MDU driver relative to version 3.0.0

1. Misc.

|  | Description |
| --- | --- |
| 1 | Based on chipset vendor driver v7_12_r886+4. |

Version 3.0.0	2022/12/28

Following is a list of new features and bugs fixed introduced in version 3.0.0 of the EoC Coax MDU driver relative to version 2.0.6

1. Misc.

|  | Description |
| --- | --- |
| 1 | Based on chipset vendor driver v7_12_r886+2. |

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | To replace sniffer. |

Version 2.0.8j	2023/07/10

Following is a list of new features and bugs fixed introduced in version 2.0.8j of the EoC Coax MDU driver relative to version 2.0.6

1. Misc.

|  | Description |
| --- | --- |
| 1 | To support XL-GHW201J & XL-GH201J with add-on default vendor notches 70~200MHz. |
| 2 | Co-work with system firmware v3.62 |

Version 2.0.6	2022/03/31

Following is a list of new features and bugs fixed introduced in version 2.0.6 of the EoC Coax MDU driver relative to version 2.0.0

1. Bugs Fixed

|  | Description |
| --- | --- |
| 1 | To replace sniffer to fix a bug that affect EoC link while handling UDP broadcast. |

Version 2.0.4 (CUSID=113)	2020/01/20

Following is a list of new features and bugs fixed introduced in version 2.0.4 of the EoC Coax MDU driver relative to version 2.0.0.

1. Misc.

|  | Description |
| --- | --- |
| 1 | The running frequency band spans over 75MHz~200MHz only (CUSID=113). |
| 2 | Vendor Notch 200~275MHz is a must (CUSID=113). |

Version 2.0.3 (CUSID=113)	2020/01/13

Following is a list of new features and bugs fixed introduced in version 2.0.3 of the EoC Coax MDU driver relative to version 2.0.0.

1. Misc.

|  | Description |
| --- | --- |
| 1 | Based on chipset vendor driver v7_10_r13+2 with extra vendor notches, the running frequency band spans over 110MHz~200MHz only (CUSID=113). |
| 2 | Co-work with system firmware v3.13 |

Version 2.0.0	2019/12/26

Following is a list of new features and bugs fixed introduced in version 2.0.0 of the EoC Coax MDU driver relative to version 1.4.4.

1. Misc.

|  | Description |
| --- | --- |
| 1 | Based on chipset vendor driver v7_10_r13+2. |
| 2 | Co-work with system firmware v3.10 |

Version 1.4.4	2019/07/08

Following is a list of new features and bugs fixed introduced in version 1.4.4 of the EoC Coax MDU driver relative to version 1.4.3.

1. Misc.

|  | Description |
| --- | --- |
| 1 | To support XL-GHW201 that could work with maximum 16 EP concurrently. To support XL-GHW2012 dual Masters (maximum 32 EP), there exists Master#1 & Master#2. |
| 2 | To support XL-GH201 EP, be as EP#1. To support XL-GH203 EP, be as EP#2. XL-GH203 has built-in WiFi 802.11ac AP. |
| 3 | This driver works with XL-GHW201 & XL-GHW2012 system firmware V3.00 and following. |

Version 1.4.3	2019/03/13

Following is a list of new features and bugs fixed introduced in version 1.4.3 of the EoC Coax MDU driver relative to version 1.4.2.

1. Misc.

|  | Description |
| --- | --- |
| 1 | Based on chipset vendor driver v7_6_r589+19+1. |
| 2 | Latency improvements. Qos engine scheduling algorithm has changed to guarantee a maximum latency of 10ms on networks having a maximum of 8 Eps. In networks with more than 8 Eps, QoS Engine will not guarantee 10 ms latency. |

Version 1.4.2	2019/01/31

Following is a list of new features and bugs fixed introduced in version 1.4.2 of the EoC Coax MDU driver relative to version 1.4.1.

1. Misc.

|  | Description |
| --- | --- |
| 1 | Start to support dual-Master model XL-GHW2012. |

Version 1.4.1	2019/01/08

Following is a list of new features and bugs fixed introduced in version 1.4.1 of the EoC Coax MDU driver relative to version 1.4.0.

1. Misc.

|  | Description |
| --- | --- |
| 1 | Co-work with system firmware v2.04 to adjust MAC address aging timer. |

Version 1.4.0	2018/12/24

Following is a list of new features and bugs fixed introduced in version 1.4.0 of the EoC Coax MDU driver relative to version 1.3.0.

1. Misc.

|  | Description |
| --- | --- |
| 1 | Based on chipset vendor driver v7_6_r589+18. |
| 2 | Change DOMAIN_ID_MODE from 5 to 1, to support future multiple Masters |

Version 1.3.0	2018/11/13

Following is a list of new features and bugs fixed introduced in version 1.3.0 of the EoC Coax MDU driver relative to version 1.2.0.

1. Misc.

|  | Description |
| --- | --- |
| 1 | Based on chipset vendor driver v7_6_r589+15. |

Version 1.2.0	2018/09/27

Following is a list of new features and bugs fixed introduced in version 1.2.0 of the EoC Coax MDU driver relative to version 1.0.1.

1. Misc.

|  | Description |
| --- | --- |
| 1 | Based on chipset vendor driver v7_6_r589+12. |

Version 1.0.1	2018/09/17

Following is a list of new features and bugs fixed introduced in version 1.0.1 of the EoC Coax MDU driver relative to version 1.0.0.

1. Misc.

|  | Description |
| --- | --- |
| 1 | Based on chipset vendor driver v7_6_r589+11. |

Version 1.0.0	2018/08/30

Initial release.

1. Misc.

|  | Description |
| --- | --- |
| 1 | Based on chipset vendor driver v7_6_r589+5. |